User’s guide for program cont_time

Karsten Fischer, Becker Technologies GmbH, Eschborn, Germany

November 2013

The program cont_time calculates the contact time distribution at the surface of a fluid subject to an incompressible stationary flow distribution. The contact time or age of the surface fluid is the time span of contact between a fluid particle and the overlaying atmosphere.

Generally, the 2-dimensional incompressible surface flow field has locations of positive and negative divergence. Positive divergence means that fluid rises from the bulk fluid below the surface and acts as source in the 2-d flow field. Negative divergence gives the sink where fluid dives back down to the bulk.

Basic equations

In this program, the age of the surface fluid is considered as a transportive property which travels with the velocity field from the sources (where the age has the value a = 0) to the sinks (where it is removed from the 2-d computational domain). While travelling along the surface, the fluid age increases corresponding to the wall clock time t. The development of the fluid age in time and 2-dimensional space is thus governed by the differential equation

(1)	∂a∂t+u∇a=−∇⋅ua+1

This equation is solved by means of a finite difference scheme with equidistant spatial mesh. Upwind differences are applied for the advective terms. A simple explicit forward time difference is used to calculate the age distribution at the new time level t + ∆t:

(2)	a(t+Δt)=a(t)+Δt−u∇a(t)−∇⋅ua(t)+1

The fluid age is used to determine the local value of the mass transfer coefficient kw

(3)	kw=Dπa

Averaging the expression 1a over the surface gives a weighted average contact time which can be used to calculate the average mass transfer coefficient.

Spatial discretization

The spatial discretization is illustrated in the following figure.

north

east

south

west

i+1,j

i,j+1

i,j

i+1,j+1

∆x

∆y

i,j

The numbering of mesh nodes and cells follows the coordinate axes. The velocity components u and v are defined at the mesh nodes; the age variable a is defines at the cell centers. As an example, the normal velocity at the east edge of the cell is given by (ui+1,j+ui+1,j+1)/2.

Data files

In order to facilitate pre- and postprocessing, data files are read and written by cont_time according to the Tecplot format specifications (see Tecplot 360 2013 Data Format Guide Release 1. Tecplot, Inc.,Bellevue, WA, 2013). The corresponding code passages should be adapted by the user to his specific needs.

Flow of calculation

The code reads first the mesh node coordinates x, y and the velocity components u, v as Tecplot file format from standard input. It then calculates the divergence q=∇⋅u for each cell. The initial value of the age a is set to a = 0.

From the maximum velocity and the cell dimensions, suitable values of the time step ∆t and the total simulation time are estimated. The time step must be small enough to keep the Courant number below 1 everywhere; this is necessary to ensure stability of the solution procedure. The total simulation time should be large enough to allow several passages of fluid particles from sources to sinks in the system and reach stationary conditions for the distribution of a.

Equation (2) is integrated over time by iterative solution. After each iteration, a selected set of results is printed on standard output for monitoring the approach to stationary conditions. It is worth mentioning that cells with stagnant flow will not reach stationary results but show continuous increase of the age parameter a. These cells are excluded from the calculation of the surface averaged contact time.

A file conttime.dat is generated which allows postprocessing the results by using Tecplot. The weighted average contact time is printed on standard output.

Copyright

The program cont_time and this user’s guide are protected by the creative commons CC BY 3.0 copyright. You are free:

to Share — to copy, distribute and transmit the work

to Remix — to adapt the work

to make commercial use of the work

under the following conditions:

Attribution — You must attribute the work (but not in any way that suggests that he endorses you or your use of the work) in the following manner:

Author Karsten Fischer, Becker Technologies GmbH, Eschborn, Germany